SPI Master & Slave IP
Specification Document

Full-Duplex Serial Peripheral Interface

Version 1.0

# Table of Contents

1. Overview

1.1 IP Introduction

1.2 Feature List

2. Interfaces

2.1 SPI Master Port List

2.2 SPI Slave Port List

3. Register Description (SFR)

3.1 SPI Master Registers

3.2 SPI Slave Registers

4. Functional Description

4.1 Block Diagram

4.2 SPI Master Design Details

4.3 SPI Slave Design Details

4.4 SPI Modes

4.5 Data Transfer Timing

5. Integration Guide

5.1 Clock and Reset

5.2 SPI Bus Connection

5.3 Register Interface Mapping

6. Known Limitations

# 1. Overview

## 1.1 IP Introduction

This document describes the specification of the SPI (Serial Peripheral Interface) Master and Slave IP cores. The SPI bus is a synchronous, full-duplex serial communication interface commonly used for short-distance communication between microcontrollers and peripheral devices such as sensors, ADCs, DACs, and memory chips.

The IP pair comprises an SPI Master module that generates the serial clock (SCLK) and chip-select (CS_N) signals, and an SPI Slave module that responds to the master's clock and chip-select. Both modules support 8-bit data transfers with MSB-first bit ordering.

## 1.2 Feature List

8-bit full-duplex data transfer (MSB-first)

Support for 4 SPI modes via CPOL/CPHA configuration inputs

Programmable clock divider (8-bit clk_div) for Master SCLK generation

Master: 4-state FSM (IDLE → LEADING → TRAILING → DONE)

Slave: Edge-detection based architecture with automatic mode selection

Active-low asynchronous reset (rst_n)

Master: Single-cycle done pulse upon transfer completion

Slave: Single-cycle rx_valid pulse upon 8-bit reception

Slave supports immediate MISO drive on CS_N assertion for CPHA=0 mode

Single system clock domain (clk) for both Master and Slave

Slave minimum SCLK half-period requirement: clk_div ≥ 2 (clk_div=1 not supported by Slave)

# 2. Interfaces

This section describes the port interfaces of the SPI Master and Slave modules. All signals are synchronous to the system clock (clk) domain unless otherwise noted.

### 2.1 SPI Master Port List

| Port Name | Direction | Width | Clock Domain | Reset Domain | Description |
| --- | --- | --- | --- | --- | --- |
| Clock & Reset |  |  |  |  |  |
| clk | Input | 1-bit | clk | - | System clock |
| rst_n | Input | 1-bit | - | rst_n | Active-low asynchronous reset |
| SPI Bus |  |  |  |  |  |
| sclk | Output | 1-bit | clk | rst_n | SPI serial clock, generated by master |
| mosi | Output | 1-bit | clk | rst_n | Master-Out Slave-In data line |
| miso | Input | 1-bit | clk | - | Master-In Slave-Out data line |
| cs_n | Output | 1-bit | clk | rst_n | Active-low chip select |
| Control |  |  |  |  |  |
| start | Input | 1-bit | clk | - | Start transfer pulse; asserted for 1 cycle to begin an 8-bit transfer |
| cpol | Input | 1-bit | clk | - | Clock polarity configuration (must be stable during operation) |
| cpha | Input | 1-bit | clk | - | Clock phase configuration (must be stable during operation; note: declared but not used in current RTL) |
| clk_div | Input | 8-bit | clk | - | Clock divider value; SCLK half-period = (clk_div + 1) system clock cycles |
| done | Output | 1-bit | clk | rst_n | Transfer complete indicator; pulses high for 1 cycle after 8-bit transfer |
| Data |  |  |  |  |  |
| tx_data | Input | 8-bit | clk | - | Transmit data; latched on start assertion |
| rx_data | Output | 8-bit | clk | rst_n | Received data; valid when done is asserted |

### 2.2 SPI Slave Port List

| Port Name | Direction | Width | Clock Domain | Reset Domain | Description |
| --- | --- | --- | --- | --- | --- |
| Clock & Reset |  |  |  |  |  |
| clk | Input | 1-bit | clk | - | System clock |
| rst_n | Input | 1-bit | - | rst_n | Active-low asynchronous reset |
| SPI Bus |  |  |  |  |  |
| sclk | Input | 1-bit | clk | - | SPI serial clock from master |
| mosi | Input | 1-bit | clk | - | Master-Out Slave-In data line |
| miso | Output | 1-bit | clk | rst_n | Master-In Slave-Out data line |
| cs_n | Input | 1-bit | clk | - | Active-low chip select from master |
| Control |  |  |  |  |  |
| cpol | Input | 1-bit | clk | - | Clock polarity configuration (must be stable during operation) |
| cpha | Input | 1-bit | clk | - | Clock phase configuration (must be stable during operation) |
| rx_valid | Output | 1-bit | clk | rst_n | Receive-valid pulse; high for 1 cycle after 8 bits received |
| Data |  |  |  |  |  |
| tx_data | Input | 8-bit | clk | - | Transmit data; loaded into shift register on CS_N falling edge |
| rx_data | Output | 8-bit | clk | rst_n | Received data; valid when rx_valid is asserted |

# 3. Register Description (SFR)

This section describes the Special Function Registers (SFR) of the SPI Master and Slave modules. These registers correspond to port-level signals that control the operating mode, initiate transfers, provide transmit data, and return receive data and status information. In a typical SoC integration, these SFRs would be mapped to bus-addressable registers.

## 3.1 SPI Master Registers

### 3.1.1 Control Registers

| Register Name | Width | Reset Value | R/W | Description |
| --- | --- | --- | --- | --- |
| cpol | 1-bit | - | R/W | Clock Polarity. Determines SCLK idle level. 0: SCLK idle low (Mode 0/1) 1: SCLK idle high (Mode 2/3) Must remain stable throughout a transfer. |
| cpha | 1-bit | - | R/W | Clock Phase. Declared as input but not used in the current Master RTL. Reserved for future use. Must remain stable throughout a transfer. |
| clk_div | 8-bit | User-defined | R/W | Clock Divider. Sets the SCLK half-period to (clk_div + 1) system clock cycles. SCLK frequency = f_clk / (2 * (clk_div + 1)). Note: clk_div = 1 is not supported by the Slave (minimum clk_div = 2). |
| start | 1-bit | 0 | W | Start Transfer. Write 1 to initiate an 8-bit SPI transfer. Self-clearing (pulse for 1 system clock cycle). Must not be asserted while a transfer is in progress. |

### 3.1.2 Data Registers

| Register Name | Width | Reset Value | R/W | Description |
| --- | --- | --- | --- | --- |
| tx_data | 8-bit | User-defined | W | Transmit Data. The 8-bit data to be shifted out on MOSI (MSB-first). Latched into the internal shift register when start is asserted. Must be valid at the time start is written. |
| rx_data | 8-bit | 8'h00 | R | Receive Data. Contains the 8-bit data received from MISO. Updated at the end of a transfer (TRAILING state, last bit). Valid when done = 1. Remains stable until the next transfer completes. |

### 3.1.3 Status Registers

| Register Name | Width | Reset Value | R/W | Description |
| --- | --- | --- | --- | --- |
| done | 1-bit | 0 | R | Transfer Done. Pulses high for 1 system clock cycle after the 8-bit transfer is complete (DONE_ST state). Automatically clears to 0 on the next clock cycle. When done = 1, rx_data contains valid received data. |

## 3.2 SPI Slave Registers

### 3.2.1 Control Registers

| Register Name | Width | Reset Value | R/W | Description |
| --- | --- | --- | --- | --- |
| cpol | 1-bit | - | R/W | Clock Polarity. Must match the Master's CPOL setting. Used together with CPHA to select sample and shift edges. Must remain stable throughout a transfer. |
| cpha | 1-bit | - | R/W | Clock Phase. Selects which SCLK edge is used for data sampling. CPOL^CPHA = 0: sample on SCLK posedge, shift on SCLK negedge CPOL^CPHA = 1: sample on SCLK negedge, shift on SCLK posedge When CPHA=0, MISO is driven immediately on CS_N falling edge. Must remain stable throughout a transfer. |

### 3.2.2 Data Registers

| Register Name | Width | Reset Value | R/W | Description |
| --- | --- | --- | --- | --- |
| tx_data | 8-bit | User-defined | W | Transmit Data. The 8-bit data to be shifted out on MISO (MSB-first). Loaded into the internal shift register on CS_N falling edge. For CPHA=0 mode, tx_data[7] is driven onto MISO immediately. Must be valid before CS_N assertion. |
| rx_data | 8-bit | 8'h00 | R | Receive Data. Contains the 8-bit data received from MOSI. Updated when the 8th bit is sampled (bit_cnt reaches 7). Valid when rx_valid = 1. Remains stable until the next 8-bit reception. |

### 3.2.3 Status Registers

| Register Name | Width | Reset Value | R/W | Description |
| --- | --- | --- | --- | --- |
| rx_valid | 1-bit | 0 | R | Receive Valid. Pulses high for 1 system clock cycle after 8 bits have been received from MOSI. Automatically clears to 0 on the next clock cycle. When rx_valid = 1, rx_data contains valid received data. |

# 4. Functional Description

## 4.1 Block Diagram

The following tables describe the sub-module decomposition and signal connectivity for the SPI Master and Slave.

#### SPI Master Sub-modules

| Sub-module | Function | Key Signals |
| --- | --- | --- |
| FSM Controller | 4-state state machine controlling the transfer sequence | state, start, done |
| Clock Divider | Generates SCLK half-period timing from clk_div | clk_cnt, clk_div, sclk |
| TX Shift Register | Parallel-load, serial-out (MSB-first) shift register | shift_out, tx_data, mosi |
| RX Shift Register | Serial-in, parallel-out shift register for received data | shift_in, miso, rx_data |
| Bit Counter | Counts 8 bits (7 down to 0) per transfer | bit_cnt |
| CS_N Controller | Drives chip-select: deasserted in IDLE/DONE, asserted during transfer | cs_n, state |

#### SPI Slave Sub-modules

| Sub-module | Function | Key Signals |
| --- | --- | --- |
| Edge Detector | Detects SCLK rising/falling edges and CS_N falling edge using 1-cycle delay registers | sclk_d, cs_n_d, sclk_posedge_w, sclk_negedge_w, cs_n_falledge |
| Mode Selector | Selects sample_edge and shift_edge based on CPOL^CPHA | cpol, cpha, sample_edge, shift_edge |
| TX Shift Register | Parallel-load on CS_N fall, serial-out (MSB-first) on shift_edge | shift_out, tx_data, miso |
| RX Shift Register | Serial-in on sample_edge, parallel capture to rx_data after 8 bits | shift_in, mosi, rx_data, rx_valid |
| Bit Counter | Counts 0 to 7 per 8-bit frame; wraps and triggers rx_valid | bit_cnt |
| Active Controller | Manages active flag: set on CS_N fall, cleared on CS_N deassert | active, cs_n, cs_n_falledge |

## 4.2 SPI Master Design Details

### 4.2.1 FSM Description

The SPI Master uses a 4-state Finite State Machine (FSM) to control the transfer sequence. The FSM states and transitions are described below.

#### Master FSM State Transition Table

| Current State | Condition | Next State | Actions |
| --- | --- | --- | --- |
| IDLE (2'b00) | start == 1 | LEADING | Load tx_data → shift_out; clear shift_in; bit_cnt ← 7; clk_cnt ← 0; cs_n ← 0; mosi ← tx_data[7]; sclk ← cpol |
| IDLE (2'b00) | start == 0 | IDLE | sclk ← cpol; cs_n ← 1 (bus idle) |
| LEADING (2'b01) | clk_cnt == clk_div | TRAILING | clk_cnt ← 0; sclk ← cpol ^ 1 (toggle); shift_in ← {shift_in[6:0], miso} (sample MISO) |
| LEADING (2'b01) | clk_cnt != clk_div | LEADING | clk_cnt ← clk_cnt + 1 (wait) |
| TRAILING (2'b10) | clk_cnt == clk_div AND bit_cnt == 0 | DONE_ST | clk_cnt ← 0; sclk ← cpol; shift_out ← {shift_out[6:0], 0}; rx_data ← shift_in (capture final data) |
| TRAILING (2'b10) | clk_cnt == clk_div AND bit_cnt != 0 | LEADING | clk_cnt ← 0; sclk ← cpol; shift_out ← {shift_out[6:0], 0}; mosi ← shift_out[6]; bit_cnt ← bit_cnt - 1 |
| TRAILING (2'b10) | clk_cnt != clk_div | TRAILING | clk_cnt ← clk_cnt + 1 (wait) |
| DONE_ST (2'b11) | (unconditional) | IDLE | cs_n ← 1; done ← 1 (single pulse); sclk ← cpol |

### 4.2.2 Clock Divider

The clock counter (clk_cnt) increments each system clock cycle during LEADING and TRAILING states. When clk_cnt reaches clk_div, the SCLK edge is generated and clk_cnt resets to 0. The resulting SCLK frequency is:

f_SCLK = f_clk / (2 × (clk_div + 1))

### 4.2.3 Shift Registers

TX Shift Register (shift_out): Loaded with tx_data[7:0] when start is asserted. On each TRAILING→LEADING transition, the register shifts left by 1 bit (shift_out = {shift_out[6:0], 1'b0}), and the new MSB (shift_out[6] before shift) is driven onto MOSI.

RX Shift Register (shift_in): Cleared to 0 when start is asserted. On each LEADING→TRAILING transition, the register shifts left and captures MISO into the LSB (shift_in = {shift_in[6:0], miso}). After 8 bits, the value is copied to rx_data.

### 4.2.4 CS_N Control

CS_N is deasserted (high) in the IDLE state when no transfer is active. It is asserted (low) on the same cycle as the IDLE→LEADING transition and remains asserted through the LEADING and TRAILING states. It is deasserted in the DONE_ST state.

## 4.3 SPI Slave Design Details

### 4.3.1 Edge Detection

The Slave uses 1-stage delay registers (sclk_d, cs_n_d) to detect edges on the SPI bus signals. Since the Master generates SCLK synchronously to the same system clock, no multi-stage synchronizer is needed.

| Signal | Detection Logic |
| --- | --- |
| sclk_posedge | sclk & ~sclk_d |
| sclk_negedge | ~sclk & sclk_d |
| cs_n_falledge | ~cs_n & cs_n_d |

### 4.3.2 Mode Selection

The slave supports all 4 SPI modes through the CPOL and CPHA configuration inputs. The sample and shift edges are selected by the XOR of CPOL and CPHA:

| CPOL^CPHA | Value | Sample Edge | Shift Edge |
| --- | --- | --- | --- |
| 0 | Mode 0 (0,0) / Mode 3 (1,1) | SCLK posedge | SCLK negedge |
| 1 | Mode 1 (0,1) / Mode 2 (1,0) | SCLK negedge | SCLK posedge |

### 4.3.3 Slave Operation Sequence

The SPI Slave operates as follows:

1. CS_N deasserted (high): Slave is idle. bit_cnt = 0, active = 0.

2. CS_N falling edge detected: active ← 1; shift_out ← tx_data. If CPHA=0, immediately drive miso ← tx_data[7] and left-shift shift_out.

3. Active & sample_edge: Shift MOSI into shift_in LSB. Increment bit_cnt. When bit_cnt reaches 7: rx_data ← {shift_in[6:0], mosi}; rx_valid ← 1; bit_cnt wraps to 0.

4. Active & shift_edge: Drive miso ← shift_out[7]; left-shift shift_out.

5. CS_N deasserted: active ← 0; bit_cnt ← 0. Slave returns to idle.

### 4.3.4 Active Flag Control

The active flag is set to 1 on the CS_N falling edge and cleared to 0 when CS_N is deasserted. All shift register and bit counter operations are gated by the active flag to prevent spurious operation when the slave is not selected.

## 4.4 SPI Modes

SPI defines 4 modes based on two parameters: Clock Polarity (CPOL) and Clock Phase (CPHA).

| SPI Mode | CPOL | CPHA | SCLK Idle | Data Sampling |
| --- | --- | --- | --- | --- |
| Mode 0 | 0 | 0 | Low | Rising edge (leading) |
| Mode 1 | 0 | 1 | Low | Falling edge (trailing) |
| Mode 2 | 1 | 0 | High | Falling edge (leading) |
| Mode 3 | 1 | 1 | High | Rising edge (trailing) |

Note: The current Master RTL declares CPHA as an input but only uses CPOL for SCLK polarity control. The Master effectively operates in Mode 0 (CPOL=0) or Mode 2 (CPOL=1) behavior regardless of CPHA. The Slave correctly uses both CPOL and CPHA to select sample and shift edges.

## 4.5 Data Transfer Timing

The following describes the timing relationship of a complete 8-bit transfer (assuming Mode 0: CPOL=0, CPHA=0, clk_div=N):

Master: start asserted → IDLE→LEADING; cs_n←0; mosi←tx_data[7]; bit_cnt←7

LEADING state: clk_cnt counts 0..N; at N: sclk←1 (rising), sample miso into shift_in

TRAILING state: clk_cnt counts 0..N; at N: sclk←0 (falling), shift shift_out, drive next mosi bit, bit_cnt--

Repeat LEADING↔TRAILING for 8 bits (bit_cnt: 7→6→...1→0)

After last TRAILING (bit_cnt==0): rx_data←shift_in; state→DONE_ST

DONE_ST: cs_n←1; done←1 (1 cycle); state→IDLE

Total transfer time: 2 × 8 × (clk_div + 1) + 2 system clock cycles

# 5. Integration Guide

## 5.1 Clock and Reset

Both SPI Master and Slave require a common system clock (clk) and an active-low asynchronous reset (rst_n). In a typical SoC integration:

Connect the system bus clock to both modules' clk input.

Connect the system reset (active-low) to both modules' rst_n input.

Ensure rst_n meets the minimum assertion width for proper initialization.

After reset deassertion, all internal registers are in their default states.

## 5.2 SPI Bus Connection

Connect the SPI bus signals between Master and Slave as follows:

| Master Port | Direction | Slave Port |
| --- | --- | --- |
| sclk (output) | → | sclk (input) |
| mosi (output) | → | mosi (input) |
| miso (input) | ← | miso (output) |
| cs_n (output) | → | cs_n (input) |

## 5.3 Register Interface Mapping

For SoC integration with a bus interface (e.g., APB, AHB-Lite), the following signals should be mapped to addressable registers:

| Offset | Name | Access | Mapped Signal(s) |
| --- | --- | --- | --- |
| 0x00 | SPI_CR (Control) | R/W | start, cpol, cpha, clk_div |
| 0x04 | SPI_TX (TX Data) | W | tx_data[7:0] |
| 0x08 | SPI_RX (RX Data) | R | rx_data[7:0] |
| 0x0C | SPI_SR (Status) | R | done / rx_valid |

Note: The register map above is a suggested mapping. The actual implementation depends on the bus protocol used for integration.

# 6. Known Limitations

The following limitations apply to the current version of the SPI Master and Slave IP:

1. Fixed 8-bit transfer width: Both Master and Slave support only 8-bit data frames. Variable-width transfers (e.g., 16-bit, 32-bit) are not supported.

2. Slave does not support clk_div = 1: The SPI Slave uses 1-cycle delay registers (sclk_d, cs_n_d) for edge detection. When the Master is configured with clk_div = 1 (SCLK half-period = 2 system clocks), the SCLK transitions are too fast for the Slave to reliably detect edges and complete the sample/shift operations within the available time window. The minimum supported value is clk_div = 2 (SCLK half-period = 3 system clocks). clk_div = 0 is also not supported as the SCLK would toggle every clock cycle.

3. No FIFO or DMA support: There is no transmit or receive FIFO buffer. Each transfer requires individual software intervention via the start signal and reading rx_data.

4. Master does not implement CPHA: The Master RTL declares cpha as an input port but does not use it in the logic. The Master always samples MISO on the LEADING edge and shifts MOSI on the TRAILING edge, effectively operating as Mode 0 (CPOL=0) or Mode 2 (CPOL=1) only.

5. Single-slave topology: The Master has a single CS_N output. Multi-slave configurations require external decode logic for additional chip-select lines.

6. No error detection: There is no overrun, underrun, or frame-error detection. If the software does not read rx_data before the next transfer, data will be silently overwritten.

7. No hardware flow control: There is no busy/ready handshaking mechanism. The host must ensure it does not assert start during an ongoing transfer.

8. Synchronous SPI bus assumption: The Slave uses single-stage delay registers for edge detection, assuming SCLK and CS_N are synchronous to the system clock. If the Master operates on a different clock domain, multi-stage synchronizers must be added externally.

9. No interrupt support: The done and rx_valid signals are simple pulse outputs. Integration of interrupt capability requires additional logic in the SoC wrapper.